SM590i-II-WX Architectural Specifications

The loudspeaker shall consist of a 133 mm (5.25 in) low-frequency transducer and a 25 mm (1 in) high-frequency transducer. The low-frequency voice coil diameter shall be 25 mm (1 in).

Performance specifications of a typical production unit shall be as follows: Useable frequency response shall extend from 69 Hz – 22 kHz (-10 dB, half space, independently verified). Measured sensitivity (2.83-volt input, 1 meter) shall be at least 87 dB. The speaker shall have a nominal impedance of 8 ohms and shall be available for 25/70.7/100-volt modes with transformer bypass. The frequency-dividing network shall have a crossover frequency of 3 kHz with a slope of 12 dB per octave (second order) for the high-pass filter. Rated power capacity shall be at least 80 watts continuous (RMS), and conforms to EIA-426-B testing. Maximum continuous output at 1 meter shall be 106 dB.

The low-frequency transducer shall have a polypropylene cone with butyl rubber surround. The high-frequency transducer shall be constructed of titanium material and incorporate a high-SPL horn.

Installation for the speaker shall be by the new Stealth™ bracket mounting system. A tool-free system with locking mounting plate for either wall or pole mount applications shall be used. For safety redundancy, a secondary cable shall be included and attached to the speaker chassis via an included 1/4-20 UNC bolt. The external wiring input connector shall be a four-pin, 5.08 mm Euroblock for 8 ohm or distributed systems and shall accept from 10 – 22-gauge wire. The system shall be for indoor/outdoor applications and shall have a weather-resistant terminal boot covering all wire connectors. Unit is IP44 approved.

The enclosure shall be constructed of injection-molded, glass-reinforced ABS. The grille shall be constructed of iridite-plated, powder-coated aluminum for lasting performance. Overall cabinet dimensions shall be no more than 289.6 mm (11.40 in) in height by 212.3 mm (8.36 in) in width by 266.2 mm (10.48 in) in depth (including SM bracket). The unit shall include mounting hardware, safety cable, Euroblock connector and terminal weather boot.

The system shall be the SoundTube SM590i-II-WX with mounting hardware for both low- and high-impedance applications.