MA501t
Architectural Specifications

The amplifier shall be a compact 50 watt, monaural, constant-voltage amplifier.

The amplifier shall provide balanced input via Euroblock and an unbalanced RCA line level input, with an input selection switch. It shall incorporate an audio grade toroidal output transformer. Outputs shall be presented on Euroblock and shall provide connections for ground plus 25V, 70V and 100V applications. Input adjustment shall be made via recessed screw gain control. The amplifier shall provide power and peak output / clipping indication via LEDs. A selectable high pass filter shall be included.

Performance specifications of a typical production unit shall be as follows: 50 watts RMS (30 Hz – 20 kHz) at >1% THD. The high pass filter shall have a sub-in crossover frequency of 120 Hz and a slope of 12 dB per octave.

An external power supply shall provide 32V DC input to the amplifier. The amplifier shall also provide a 32V DC power input via Euroblock. The amplifier shall activate either on signal sensing (auto-on) or via an external trigger ranging from 12 to 32 V DC. Triggering shall be enabled via a jumper on the exterior of the chassis. 

Construction of the chassis shall be extruded aluminum. Mounting options shall include pole mount, rack mount or chassis mount. Overall cabinet dimensions shall not exceed 30.5 mm (1.2 in) height by 99.1 mm (3.9 in) width by 203.2 mm (8.0 in) depth. 

The system shall be the SoundTube MA501t.